
[image: image1.png]/. .
£





Hazard Register

Version 2025
Edit as appropriate

	Task
	Hazards
	Controls

	Chainsaw Use
	1. Noise and vibration.
	· All users MUST be inducted by NEC or NEC nominated contractor prior to use of Chainsaws

· Always wear grade four earmuffs

· Chainsaw maintenance



	
	2. Cutting yourself, others or equipment.
	· Wear all safety equipment as listed in COP For Safety and Health in Forest Operations.

· Operator training

	
	
	· Correct maintenance

· Always cut away from your body and equipment.

· Always use two hands on the saw.

· Firm stance before cutting.

· Ensure saw has all safety features working before use.

· No person shall enter a 3m radius of a saw being used.



	
	3. Dust or wood particles in eyes
	· Always wear eye protection.



	
	4. Fire during refuelling.
	· Do not start saw within 3m of refuelling site

· Full oil tank first.

	Tree Felling
	1. Tree falling on feller, traffic or other persons.

2. Electrical conductors
	· Use of correct techniques for situation.

· Use felling aids.

· No other persons to enter a 2 tree length radius of the tree being felled.

· No felling within two tree lengths of any public road unless traffic is stopped.

· Warning signs, tape and staff to isolate area.

· No felling of trees within two tree lengths of conductors unless a suitable pulling line is attached

· Always list conductors present on Site Specific Hazard Identification Plan.

· Any doubt dismantle tree.



	Tree

Climbing


	1. Tree hazards e.g. hangers, decay etc.

2. Climbing equipment failure.

3. Falling debris.

4. Electrical conductors.

5. Falls.

6. Injured climber unable to descend.
	· Training.

· Visually inspect tree before climbing.

· List specific hazards on Site Specific Hazard Management Plan with control measures.

· Visual inspection before use (daily).

· Weekly inspection (recorded).

· Training in correct use.

· All equipment must have a tensile strength of 2200kg and above.

· All connections must be triple locking i.e. carabineers

· Wear approved hardhat.

· Inspect tree before climbing.

· Inspect tree and site before work starts, list location of conductors on Site Specific Hazard Management Plan.

· Maintain Minimum Approach Distances (MAD’s) as listed in COP.

· Inspect tree for defects before climbing.

· Training in safe climbing practices.

· Maintain at least one closed point of attachment to the tree at all times i.e. rope or lanyard.

· Select strongest practical roping point.

· At least one member of ground staff must be trained in aerial rescue procedures, have sufficient equipment, and be capable to perform an aerial rescue if the climber is working above 12 metres (excluding tree inspections), or carrying out more than simple tasks.



	Pruning, Dismantling & Rigging 


	1. Cutting ropes, yourself and equipment.

2. Branches falling on fences, structures, underground services or electrical conductors.
	· Always cut away from body and equipment when using handsaw or chainsaw.

· Wear all protective equipment as listed in COP’s.

· Engage chain brake on chainsaw when not being used. Turn it off before changing position.

· Inspect site before starting work. List any of these hazards on the Site Specific Hazard Management Plan.

· Cut and hold branches (handsaw).

· Rope and lower any branches that the direction of fall can’t be controlled.



	
	3. Branches falling on ground staff.

4. Equipment failure.

5. Crush injuries to climber by falling limbs and loads suspended on ropes.

6. Tree failing due to loading created by rigging operations.


	· Hard hats.

· High viz clothing.

· Warn ground staff before cutting and check it is clear.

· Ground staff paying attention to climber when under tree.

· Do not stand directly under branches suspended on ropes.

· Know SWL (Safe Working Load) of equipment and do not exceed.

· Check equipment visually before use. Retire damaged equipment.

· Recorded checks weekly.

· Training

· Position yourself above and away from the rope and tree sections intended direction of fall.

· Plan work process.

· Training

· Assess trees stability to handle proposed rigging operations. List defects and control measures on the Site Specific Hazard Management Plan.

· Plan work method to reduce loading to a factor that can be safely absorbed by tree.

	Working In Public Areas
	1. Public entering worksite.

2. Traffic. 
	· Warning signs and tape

· Crew to warn public away.

· No tree felling within two tree lengths of any public road unless traffic is stopped.

· High viz vests to be worn while working on the road

· Correct cones and signs too Transit regulations.



	Any Machinery


	1. Body parts caught in moving parts or workers being struck by moving plant.

Electrical conductors
	· All users MUST be inducted by NEC or NEC nominated contractor prior to use of any machinery

· Training.  All safety guards and features are in place and working.

· Keep required safe distance from operating equipment High viz clothing

· Get operators attention and clearance before approaching machine.

· No passengers.

· Maintenance

· Wear suitable hearing protection

· Regular breaks when using this type of machinery.

· Inspect site before work starts, list location of conductors on Site Specific Hazard Management Plan.

· Keep 4m away unless permission is received from supply authority.

· If permission is obtained, maintain Minimum Approach Distances as listed in COP.



	Working Outdoors


	1. Skin cancer.

2. Bad weather.

3. Dehydration
	· Apply sunscreen, which is provided in all vehicles.

· Wear warm, waterproof clothing when wet and cold.

· Drink water regularly.



	Digging (Hand or Machine)


	1. Underground services.


	· Inspect site before starting work, list location and type of all services on Site Specific Hazard Management Plan.

· Maintain required distance from these services.



	Lifting
	1. Over exertion or strain
	· Get help to lift heavy objects.

· Cut large pieces of wood smaller into manageable lengths



	Machinery:

  ATV & Mule
	1.  Operating ATV’s and Kawasaki Mule.

2.  Driving on uneven surfaces, slippery sloping or hilly terrain.

2. Carrying loads.

3.  Overloading vehicles

4.  Poor vehicle maintenance
	· All users MUST be inducted by NEC or NEC nominated contractor prior to use of ATV or Mule

· Wear suitable strong footwear when operating machines.

· Never ride vehicles when fatigued or under the influence of alcohol.

· Use alternative routes that can be used in adverse conditions.

· Use smooth throttle applications when climbing a slope will help maintain stability.
· Wear suitable footwear (steel cap boots).
· When riding up a slope lean forward and select a suitable gear.
· When riding down a slope, move your body backwards and select a suitable gear.
· Look out for potential hazards when riding such as rocks, bumps, holes, particularly if riding through long grass or scrub.
· Nobody under the age of 16 is to operate the ATV or mule unless authorised by NEC rep.
· It is common for spray tanks to weigh up to 100kg when full.  These tanks can create an stability problem which can result in a loss of stability.  Do not fill spray tanks above the manufacturer’s recommended limit, taking into consideration the weight of the tank, its contents, brackets and attachments.
· Ensure the load is evenly distributed and well secured.

· When using attachments and carrying loads, ensure any attachment or load does not alter the ATV’s centre of gravity and affect its stability.
· Most ATV’s have carry frames front and rear.  The manufacturer’s loading for these carry frames should be noted and not exceeded.
· After riding through water, check brakes.

· Follow the manufacturer’s recommended maintenance plan and ensure all repairs are carried out by a competent person.
· Regularly check: tyre inflation, tread, brake and throttle cable condition, steering ball joints, oil and accessory attachments.



	Electric Fence Units
	1.  Working with or around electric fences.  Climbing through fences.
	· Ensure electric fences that are hidden from obvious view or across roadways are clearly tagged or marked.
· Where electric fences are positioned close to gateways and walking tracks must have signs displaying the wording “Electric Fence”.
· Ensure people working in the vicinity of hidden fences are aware of there whereabouts.
· Wear gumboots or rubber footwear when working with electric fencing to reduce shock.
· All mains (110v/230v) operated energizers must conform with the local standard requirements.
· Ensure no more than one energizer electrifies any one fence line at any one time
· Keep fencing away from combustible material, or in times of extreme fire risk, use the reduced power terminal or disconnect energizer..
· Where possible avoid any fence line passing under or running parallel to power lines.



	Hand Tools
	1.  Incorrect use of hand tools

2. Repetitive action using tools frequently.

3.  Tools left lying around.
	· All users MUST be inducted by NEC or NEC nominated contractor prior to use of any Hand Tools that can cause harm.

· Ensure you are wearing appropriate clothing and personal protective equipment, e.g. gloves, safety glasses.
· When using sharp tools for cutting never cut towards your body or hand.

· Organise work so you can mix repetitive and non-repetitive activities.
· Take frequent, short rest breaks.
· Vary or rotate jobs to avoid over use.
· Try to regularly undertake simple and gentle exercises to reduce muscle tension during rest breaks.
· Try to arrange work materials or equipment to avoid overreaching or twisting, which can place further stress on muscles.
· Hand tools for repetitive tasks should be a comfortable size, shape and weight.

· Good housekeeping practices should be part of the daily routine, with clean up being a continuous procedure.

· The work area should be kept free of wastee and loose materials as well as tools.  This is especially important around ladders, ramps, stairs and passageways.
· Sharp edged or pointed tools should have the edge or point guarded at all times when not in use.



	Plant: Pumps
	1.  Contact with hot parts.

2.  Excessive noise.

3.  Working on or near pumps


	· Lag pipes where necessary.
· Ensure the appropriate safety equipment is provided and used.

· Wear hearing protection when required.

· Do not remove guards for maintenance until the machine is stopped and isolated with all power sources neutralised.
· Ensure guarding design is practical – allows access and protects the user.
· Keep hands well clear of trapping points.
· Ensure all guards are in place before commencing work.
· You should not wear loose clothing.
· Keep hands and body well clear of all moving parts.
· Ensure long hair is tied back.



	Plant: Power tools
	1.  Working with or around power tools

2.  Excessive noise

3.  Flying objects, projectiles, sparks.

4.  Poorly maintained equipment

5.  Repetitive use of power tools

6.  Working with or around moving parts

7.  Working with vibrating equipment
	· All users MUST be inducted by NEC or NEC nominated contractor prior to use of Power Tools

· You should never operate power tools unless trained and completely familiar with the tool in question.
· Make a quick inspection of the tool before use.
· Keep power cords clear of sharp objects, oil, grease and hot surfaces.
· Always use a RCD or isolating transformer when using power tools.
· Always test  RCD before use.

· Hearing protection should be worn when working around excessive noise.

· Regularly maintain power tools and machinery.  Keep parts well oiled, change old or loose parts, vibrating mounts, silencers etc.

· Keep moving parts directed away from the body.
· Eye protection should be worn when operating tools if there is a risk of flying objects.
· If necessary use clamps to hold article firm, or ask for assistance to hold article.

· Ensure equipment is serviced regularly by a certified person.
· Damaged or worn cords should be replaced immediately.

· Organise work so you can mix repetitive and non repetitive activities.
· Take frequent short breaks.

· Keep moving parts directed away from the body.
· Wear appropriate firm fitting clothing and have long hair tied back.
· Safety guards should not be removed or modified.  Ensure equipment is fitted with correct guards.
· Use clamps or vices when to hold work wherever possible.
· Always use two hands when operating power tools.
· Ensure loose cords are not left lying around.
· Ensure chains and blades on machinery are sharpened to manufacturer’s specifications.
· Routinely check any anti vibration mounts for damage or wear, replace if necessary.
· Maintain firm grip on handles.
· Ensure job rotation or regular breaks are taken to avoid over use.



	Plant:  Tractors


	1.  Children on or around tractors

2.  Excessive noise

3.  Climbing up/climbing down

4.  Moving on

5.  Attaching and using implements

6.  Using hydraulics
7.  Protect yourself


	· All users MUST be inducted by NEC or NEC nominated contractor prior to use of Chainsaws

· Children must be 12 years old in order to be allowed to drive an agricultural tractor and 15 years old to drive it on a public road.
· It is illegal for a child under 12 to ride on an agricultural implement drawn by a tractor unless that implement is “a sled or trailer designed or adapted exclusively or principally for the carriage of passengers or goods”.
· It is illegal for a child under 12 to ride on a tractor ‘while it is drawing any implement”.
· Children are not allowed to ride on the tractor with the operator.
· It is illegal for children under the age of 15 to drive any tractor other than an agricultural tractor.

· Watch for children when driving in the farmyard, around farm buildings, and along races. 
Children under five years are especially at risk and should be supervised by an adult at all times.
· If operating a tractor without a well insulated cab hearing protection should be worn (min grade 4).

· It is generally recommended that a tractor be mounted and dismounted from the left-hand (clutch pedal) side.
· Dismounting
It is safest to dismount backwards − to stand up, move the right leg through between the steering wheel and seat, turn to face the seat while doing so, and step down backwards.
· Nearly all offset trailing equipment is mounted on the right-hand side.

· If the tractor should move after dismounting from the right-hand side there is danger of being caught by this equipment.

· This way the whole process is controlled. One hand grips the steering wheel, the other grips the mudguard.
By dismounting this way, if clothing catches, it can be disentangled before a fall occurs.

· NEVER move a tractor from rest unless you KNOW that all is clear. There could be a small animal or a young child in front of or behind you.

· There are some important points you will need to remember as a tractor driver: 

· The brake pedals are designed to be operated individually to help turn the tractor with a heavy load. Without that load though, they can be dangerous if only one brake is applied, especially at speed.

· The brakes should be kept adjusted so that when applied, they grip evenly on both wheels and the tractor does not veer off to either side.

· Inexperienced drivers should keep the brakes locked together. If either wheel is braked individually at high speed the tractor will swerve violently to one side, and could overturn, even on flat ground.

· Keep both hands on the steering wheel where practical and keep thumbs on the outside of the wheel.
· Don't turn your wrist over and grip the inside of the wheel when turning or reversing. 

· Never coast with the engine off, especially if the tractor has power steering. Steering then becomes heavy. 

· Keep your speed down to suit the surroundings and ground surface. 

· Only one person, the driver, should be allowed on the tractor when it is moving unless a passenger seat is specifically provided. 

· There are five types of hydraulically-mounted or trailed equipment:

Drawbar-trailed implements 

· It's safest to PULL FROM THE FIXED DRAWBAR (sometimes referred to as the swinging drawbar).

· Always use the correct size drawbar pin with a lynch pin through the bottom of it when attaching trailed implements. 

· Choose the correct gear − a good guide is to descend a hill in the same gear it takes to go up. 

· The use of clutch and brakes together while towing a heavy load up or down hill is not recommended. 

· Leave the clutch alone, and brake sparingly to avoid locking the wheels in a skid. 

· Drawbars attached directly to the hydraulic lift-arms called hydraulic drawbars may give more traction but should not be raised too high.

· Pulling from the ‘top link’ position can make the front of the tractor rear up, which is dangerous. 
The top link is for attaching three-point linkage implements only. 
· NEVER PULL FROM THE TOP LINK!
Three-point linkage 

· When picking up implements on the three-point linkage, reverse in low ratio up to the centre of the three-point linkage attachment and square with it.

· Reverse up to the implement to be attached so that the left-hand lower link arm (the arm without an adjusting handle on it) is beside the attaching point. 

· Park the tractor and dismount. 

· Attach left-hand side first, then use the levelling lever adjustment to wind the right-hand lower link arm to the height of the right-hand attaching point and connect. 

· When these two points are attached, then attach the ‘top link’ or ‘centre link’ to the tractor and then the implement. 

· When all three points are attached, the implement should be raised off the ground and ‘levelled’ behind the tractor by use of the right-hand lower link arm adjuster and/or adjustment of the top link or centre link. 

Power-take-off driven implements 

· This is the means by which the power of the tractor engine is used to drive trailed or mounted rotating equipment (such as hay-baler, rotary hoe, mower, forage harvester, spray pump). 
Once again there are basic rules when using PTO driven equipment:

· When attaching PTO powered implements, the tractor engine should be shut down while attaching the PTO shaft.

· Unguarded machinery shafts have caused death and injury to many adults and children. SHAFTS ON ALL PTO DRIVEN EQUIPMENT MUST BE GUARDED AT ALL TIMES. When not in use, the tractor stub shaft should be covered by both the guard and shield provided.

· Walk around the tractor and the machine first to check that no-one is likely to be hurt when the machine starts.

· Before engaging the PTO lever, ensure that there is no object in the path of the blades, flails or other moving parts of the machine to be used. Engage the PTO only from the tractor seat. 

· Many accidents involving the PTO have occurred around or near the yard and buildings where tractors or machines were left unattended.

· An operator should never adjust, service or lubricate a machine while either it, or the tractor engine, is running. 

· At ‘start-up’, use a slow engine speed; increase speed after engagement of the PTO. 

· Before leaving the seat of the tractor, disengage the PTO lever and stop the machine completely. 

· Make sure you find out the correct PTO speed and corresponding tractor engine speed at which to run the implement. Check the operator's manual. 

Rear-mounted implements 

· It is very easy to overload rear-mounted transport trays to the point where the tractor's front wheels have insufficient weight for steering and tractor stability. The driver may be unaware of the problem until a hill is climbed or even a bump is struck and the front end lifts off the ground. 
If heavy loads on the back are a regular thing, front-mounted counterweights or a front carrier are recommended. 

· In operating any equipment mounted on tractor hydraulics there are certain points which should be noted:

· The hydraulic controls should be operated only FROM THE TRACTOR SEAT. While it is time-consuming to do this, the driver's seat is the safest place from which to make height adjustments.
Tractor hydraulics are enormously powerful.
· When implements are attached ALWAYS check that hydraulic equipment is lowered to the ground and the power-take-off (PTO) is disengaged, before leaving the operator's seat.
· If anyone or anything gets caught in the moving parts, the driver or operator needs to be in a position to take immediate corrective action. 
Be aware of "pinch-points". KEEP HANDS, FINGERS AND FEET AWAY.
· Beware of tractors having two hydraulically mounted implements running on the one hydraulic system, even when the engine is not running. Sometimes the heavier implement will cause the other one to move unexpectedly.

· If you are required to work under raised equipment of any sort (such as front-end loaders, hay elevators, truck hoists) PUT A MECHANICAL SUPPORT UNDER IT FIRST! Don't rely on hydraulics or mechanical locks to protect you.

· When parking the tractor, even for a short time, ALL HYDRAULIC EQUIPMENT SHOULD BE LOWERED TO THE GROUND.
· Close fitting clothes

· The type of clothes you wear can have a big influence on your safety.

· Avoid loose-fitting or flapping clothing when working with tractors and machinery.

· Wearing overalls is recommended.

· Gumboots and working boots with steel toe caps are recommended.

· Hearing protection

· Hearing loss can accumulate without you being aware of it. When operating a tractor, hearing protection (ear muffs or ear plugs) should be worn at all times.

· Checking for debris
· If the tractor has inadequate rear-wheel mudguards, of if the wheels are extended beyond the mudguard cover, give the wheels a quick clean before accelerating to road speed after working soft ground.

· Check especially for rocks lodged between dual tyres. Debris thrown off the tyres as the machine accelerates could become deadly missiles.

· Safety frames

· Statistics prove that SAFETY FRAMES SAVE LIVES! 
In the unfortunate event of a roll over, where a safety frame is fitted, DO NOT ATTEMPT TO LEAVE THE TRACTOR SEAT OR CAB. Hold on tightly to the steering wheel − the safety frame will provide considerable protection.
Maintenance

· Preventive maintenance is the practice of maintaining equipment in good working condition.
A poorly maintained machine greatly increases the risk of injury to the operator. Failure of working parts (such as brakes) to perform their function can lead to a very dangerous situation. 
Features which should be checked on a regular basis include:

· BRAKES - lubricate linkages, and adjust brakes evenly on both sides
· TYRES - check air pressure, water content, damage, wear. Also check tightness of wheel nuts.
· HYDRAULICS - lubricate linkages and adjust
· BATTERY - top up, keep fully charged
· STEERING - lubricate, check linkage, nuts, etc
· SHIELDS AND GUARDS - keep in place, replace if damaged
· DAILY - keep fuel, water, engine oil and transmission oil topped up and check air-cleaner indicator, or pre-cleaner.


	Plant:  Welding
	1.  Arc welding
2.  Electric welding
	· All users MUST be inducted by NEC or NEC nominated contractor prior to use of all Welders

· Ensure you use a shield or helmet with a suitable grade of filtered lens.
· To protect your skin from burns, wear long sleeves, overalls and steel capped boots.
· Avoid welding cutting or heating empty drums.  Always check what has been inside an if necessary clean thoroughly before welding cutting or heating.

· Welding can also generate toxic fumes fro chemical residues.  Avoid welding on metal coated surfaces such as galvanised steel.

· Have regulators maintained by a competent person.

· Take care not to drop or damage gauges and regulators.

· Ensure equipment is in good working order.

· Do not allow any fittings of oxy-acetylene equipment to be contaminated with grease or oil under any circumstances.

· Do not oil unions, gauges or other components.

· Use the correct colour and type of hoses and fittings recommended by the manufacturers.

· Flashback arrestors should be fitted to all oxy-acetylene equipment to overcome the danger of flashback.

· Do not light up welding equipment with matches or cigarette lighters.  Use an appropriate flint electric ignition device.

· Do not smoke when welding on jobs.

· All electrical hazards should be identified and addressed.

· Always install the welding machine as close as possible to the power source.

· Always keep the welding machine terminals and cable connections clean and tight.

· Always wear rubber insulated shoes.

· Always get a qualified electrician to do any electrical repairs 

· Have a suitable fire extinguisher close for all welding cutting heating and grinding activities.



	Animals:  Sheep
	1.  Lifting sheep (dead or alive).
2.  Working around or having direct contact with animal urine, organs, offal, contaminated water and birth fluid from animals.
	· If sheep need to be lifted get assistance where possible.
· When lifting alone; sit the sheep on its rump, squat down, take a firm hold of its back legs while keeping the sheep’s head up to restrict movement.  Pull the animal firmly against your body, and lift using your legs, not your back.

· Maintain all basic hygiene precautions such as good hand washing after handling stock.

· Use disposable gloves when handling dead animals.

· Immediately burn or bury any found carcasses.

· Clean down vehicles.

	Chemicals

Chemicals
	Agri-chemicals at work

Members of the public coming into contact with chemicals.

Spraying with chemicals

Storing of chemicals

Fuels (petrol, diesel, oils)

Handling of fuels
	· It is the user’s responsibility to ensure they have adequate information available on any product which may be hazardous to themselves, other employees or other people in the arboretum.

· For more in depth information about the active ingredients in agri-chemicals a Material Safety Data Sheet (MSDS) should be used.

· Follow manufacturer’s instructions for mixing applying and disposal of the product.  Refer to the label on containers and also wear the suitable personal protective equipment listed.

· Always wash hands after using chemicals before going to the toilet, eating or smoking.  Do not eat or smoke while mixing or applying chemicals.

· Ensure staff is trained in the use of applying agrichemicals.

· Ensure that chemicals are stored out of reach in a clearly marked and locked area.

· Spray early in the morning when members of the public are not around or use signs to warn people of spraying in progress.

· If working alone advise someone where you will be working and take RT with you

· Never use the same equipment to apply herbicides and pesticides.

· Store chemicals only in their original containers and ensure the containers are closed and clearly labelled.

· Ensure suitable fire extinguishing equipment is available.

· Incompatible chemicals, i.e. those that will adversely react if mixed together, must not be stored together, refer to labels and MSDS sheets.

· In general store powders above liquids.

· All storage must be properly signed.
· Storage should be secure and lockable.

· Storage should be adequately ventilated and not subjected to excessive dampness.

· 

	Ocmis Mobile Irragators

1 large

1  small
	1.  Working with or around irrigators
2.  Excessive noise

3.  Flying objects, projectiles, sparks.

4.  Poorly maintained equipment

5.  Repetitive use of power tools

6.  Working with or around moving parts

7.  Working with vibrating equipment
	· All users MUST be inducted by NEC or NEC nominated contractor prior to use of Ocmis Mobile Irrigators
· Make an inspection of the irrigator before use.

· Keep hoses clear of sharp objects, oil, grease and hot surfaces.
· Notify all ground users prior to beginning

· Check for hanging trees or wires prior to turning on

· Check ground for safe travel of the irrigators
· Hearing protection should be worn when working around excessive noise.

· Regularly maintain power tools and machinery.  Keep parts well oiled, change old or loose parts, vibrating mounts, silencers etc.

· Keep moving parts directed away from the body.

· Eye protection should be worn when operating irrigators if there is a risk of flying objects.

· If necessary use clamps to hold article firm, or ask for assistance to hold article.

· Ensure equipment is serviced regularly by a certified person.

· Damaged or worn parts should be replaced immediately.

· Organise work so you can mix repetitive and non repetitive activities.

· Take frequent short breaks.

· Keep moving parts directed away from the body.

· Wear appropriate firm fitting clothing and have long hair tied back.

· Safety guards should not be removed or modified.  Ensure equipment is fitted with correct guards.

· Use clamps or vices when to hold work wherever possible.

· Always use two hands when operating irrigators.

· Ensure loose parts are not left lying around.

· Ensure hoses, connections on machinery are set to manufacturer’s specifications.

· Routinely check any anti vibration mounts for damage or wear, replace if necessary.

· Maintain firm grip on handles.

· Ensure job rotation or regular breaks are taken to avoid over use.

· 

	Automatic Irrigation System
	1.  Working with or around irrigators

	· All users MUST be inducted by NEC or NEC nominated contractor prior to use of the Irrigation System
· Notify all users prior to beginning
· Check grounds are clear and safe for irrigators to begin

	
	
	· 


Note: 

In addition to these control measures all work procedures must be carried out in accordance with the following Approved Codes of Practices:

· The Approved Code of Practice for Safety and Health in Tree Work:

· Part 1: Arboriculture.

These are located in the office.

I have read and understood all hazards contained in this Hazard Register:
	Name
	Position
	Signed
	Date

	
	
	
	

	……………………………
	………………………….
	………………………
	……….

	……………………………
	………………………….
	………………………
	……….

	……………………………
	………………………….
	………………………
	……….

	……………………………
	………………………….
	………………………
	……….


Last Updated 11.08.2025
administrator@nec-chch.co.nz
NEC Christchurch
Page 1
Version August 2025

